ASSEMBLY INSTRUCTIONS

Description: II151-0012

Important Safety Instructions:

- These instructions are provided for your safety. It is very important that they are read carefully and completely before assembling fixture..

- This fixture has been rated for up to (1) 60-watt maximum TYPE A bulb (no included) or (1) 13-watt compact fluorescent light bulb (not included). To avoid the risk of fire, do not exceed the recommended wattage.

# Pre-assembly:

- Do not discard any contents until after assembly is complete to avoid accidentally discarding small parts or hardware.

- Remove all parts and hardware from box along with any plastic protective packaging.

Parts Enclosed:                              Hardware Included:

# To install you will need: Screwdriver/Electrical tape

To Adjust the Suspension Length of the Fixture:

      - The adjustment or shortening of the lighting fixture’s suspension length is only needed if the factory set Power Cord (J) length is not appropriate for your installation. We recommend that you adjust the Cord (D) to the proper desired length before cutting the electrical cord.

      - Determine the lighting fixture suspension length from the ceiling for the product installation, allowing the extra cord length for the outlet junction box wire connections as noted below.

      - If need, separate the Mounting plate (A) from the Canopy (B) by removing the 2 Canopy screws (c). Do not discard the Canopy screws (c) as these will be used later.

      - Loosen the Allen set screw (d) from the Cord collar (C) using the Allen wrench (e) by turning the wrench in a counter-clockwise.

      - Insert the Cord (D) upwards through the Canopy (B) until a new desired suspension length is determined.

      - Secure the Cord (D) in place by turning the Allen set screw (d) in a clockwise direction using the Allen wrench (e) until tight. Do not over tighten the screw, making sure the Cord (D) cord will not slip downwards with the pendant body.

      - If needed, cut off the extra power cord length on the 3 wire cords so that they all are at least 6 inches (152.4 mm) long within the outlet junction box (J-box).

# Assembly Instruction:

# Step1. Shut off the main electrical supply from the main fuse box/circuit breaker!

# Step2. Gently pull wiring from ceiling outlet box and allow the wires hang

# Step3. It is imperative that the outlet box in your home be properly grounded. If you outlet does not have a GROUND WIRE, be sure to insert the green ground screw on the Mounting plate (A) facing DOWN, and wrap the fixture’s bare copper ground wire around the green ground screw on the Mounting plate (A). If your outlet box has a GROUND WIRE (green or bare wire) attach this wire to the bare copper GROUND WIRE of the fixture. Fasten with a Plastic wire connector (b) and wrap with electrical tape

# Carefully inspect the fixture’s wires. The NEUTRAL FIXTURE WIRE is White. The HOT FIXTURE WIRE is Black. And the GROUND WIRE is Green.

# Connect the NEUTRAL WIRE from the fixture to the NEUTRAL JUNCTION BOX WIRE from the outlet box (Usually white). Fasten each together with a Plastic wire connector (b) and wrap the connection with electrical tape. Be sure that no wire strands are exposed.

# Connect the HOT WIRE from the fixture to the HOT JUNCTION BOX WIRE from the outlet box (usually black). Fasten each together with a Plastic wire connector (b) and wrap the connection with the electrical tape. Be sure that no wire strands are exposed. DO NOT REVERSE THE HOT AND NEUTRAL CONNECTIONS OTHERWISE SAFETY WILL BE COMPROMISED.

# Step4. Tuck the wire connections neatly into the ceiling outlet box..

Step5. Attach Mounting plate (A) to the ceiling outlet box, aligning the slots of the Mounting plate (A) with the two threaded holes located on the J-Box. Secure the Mounting plate (A) by tightening 2 Mounting screws (a) with a screwdriver.

Step6. Raise Canopy (B) up over the Mounting plate (A), align the holes on the Mounting plate (A) with the holes on the side of the Canopy (B) Insert the 2 Canopy Screws (c) and tighten with a screwdriver.

- Step7. Unscrew the Socket ring (f) and Metal spacer (G) from the Socket (E) by rotating the Socket ring (f) in a counter-clockwise direction.

Step8. Raise the Glass shade (F) over the Socket (E). With the help of another people to hold the Glass shade (F) in place, position the Metal spacer (G) over the Socket (E) and tighten the Socket ring (f) clockwise to secure the Glass shade (F) in place. Do not over tighten.

- Step9. Insert light bulb (not include) into Socket (E)

- Sep10. Restore power to outlet box and test fixture.

Step17. Assembly is complete.

Care instructions:

• Wipe with soft dry cloth only

• Always avoid the use of harsh chemicals or abrasive as they may damage to the finish.

Thank you for your purchase