ASSEMBLY INSTRUCTIONS

Wren Pendant (Item# II151-0097)

Important Safety Instructions:

- We recommend you contact a professional electrician to install any hardwired lighting. Incorrect installation can lead to fires.

- These instructions are provided for your safety. It is very important that they are read carefully and completely before assembling fixture.

- This fixture has been rated for up to (1) 60-watt maximum TYPE A bulb (not included) or (1) 13-watt compact fluorescent light bulb (not included). To avoid the risk of fire, do not exceed the recommended wattage.

# Pre-assembly:

- Do not discard any contents until after assembly is complete to avoid accidentally discarding small parts or hardware.

- Remove all parts and hardware from box along with any plastic protective packaging.

Parts Enclosed:                                   Hardware Included:

| Item | Description | QTY |
| --- | --- | --- |
| A | Mounting Plate | 1 |
| B | Canopy | 1 |
| C | Swivel | 1 |
| D | Tube A | 1 |
| E | Tube B | 3 |
| F | Socket | 1 |
| G | Glass Shade | 1 |
| H | Lamp cup | 1 |
| I | Threaded socket ring | 1 |

| Item | Description | QTY |
| --- | --- | --- |
| a | Mounting Screws | 2 |
| b | Plastic Wire Connectors | 3 |
| c | Canopy screw | 2 |
| d | Internal slotted screw | 1 |

# To install you will need: Screwdriver/Electrical tape

To Adjust the Suspension Length of the Fixture:

      - Determine the lighting fixture suspension length from the ceiling for the product installation, allowing the extra cord length for the outlet junction box wire connections as noted below.

      - If need, separate the Mounting plate (A) from the Canopy (B) by removing the 2 Canopy Screws (c). Do not discard the Canopy Screws (c) as these will be used later.

      - Install the swivel, tube A(D), tube B(E),on the socket (F).

      - Place the glass shade (G) and making sure the socket (F) extends through the round hole of glass shade (G).

- Put the lamp cup (H) on the socket (F) and tighten it with the Threaded socket ring (I).

      - Screw the light bulb into the socket (F).

Care instructions:

• Wipe with soft dry cloth only

• Always avoid the use of harsh chemicals or abrasive as they may damage to the finish.

Thank you for your purchase