ASSEMBLY INSTRUCTIONS

Holloway Pendant (Item#: MPS151-0088)

Important Safety Instructions:

- For safety purpose, this lamp is equipped with a polarized plug (one blade is wider than the other). If the plug does not fit securely into your outlet, do not force it, contact a professional electrician. Never alter the plug in any way.

- These instructions are provided for your safety. It is very important that they are read carefully and completely before assembling fixture.

- This fixture has been rated for up to (8) 40-watt maximum TYPE B bulb (not included).To avoid the risk of fire, do not exceed the recommended wattage.

# Pre-assembly:

- Do not discard any contents until after assembly is complete to avoid accidentally discarding small parts or hardware.

- Remove all parts and hardware from box along with any plastic protective packaging.

Parts Enclosed:                                Hardwire Parts

# Warning:

- Wires and cables are all attached (further wiring is not needed from consumer). When unpacking product, be careful to not pull wires as it may result as a bad connection. Do not plug into electrical outlet until lamp is fully assembled.

# Note:

# Turn the power off (unplug from the wall outlet) before replacing the bulb. Make sure the bulb has sufficient time to cool down.

# Assembly Instruction:

# Step1. Unpack and place all parts on a surface or floor.

Step2. Put Lamp Body (E) on a Flat surface.

Step3. Put Glass Shade (H) onto Socket (F), and put Socket cap (I) onto Socket (F), Tighten with Socket Ring (G).

Step4. Screw the Tubes (D) onto the Lamp Body (E).

There are 3 sets Tubes, please assemble how many as you needed.

Step5. Put the Canopy (C) and Metal Plate (K) onto the Tubes, tighten with Butterfly Nut (B).

# To install you will need: Screwdriver/Electrical tape.

# Please ask a Professional electrician to instal this lamp

.

Care instructions:

• Wipe with soft dry cloth only

• Always avoid the use of harsh chemicals or abrasive as they may damage to the finish.

Thank you for your purchase