ISTA 3A drop test

| Customer | MP | Tested by | Simon |
| --- | --- | --- | --- |
| Vendor | XN | Report Author | Simon |
| Test Date | 2020-11-11 | Test Result | PASS |

| Item# | Package Size | Length + Girth | Product size(in.) | G.W. (KG) | N.W.(KG) |
| --- | --- | --- | --- | --- | --- |
| MP100-0575 | 32”X32”X26.75”H | 149.5” | 30*30.5*30.25" H | 27 kg | 19.5 kg |
| Description | Brayden Accent Chair |  |  |  |  |

| Test Specimen Picture | Test Result Summary |
| --- | --- |
|  | 包装方式按3A标准试摔， 测试通过 |

| Packaging picture | Product Picture |
| --- | --- |
| Failure details |  |
| N/A | N/A |
| Suggested Corrective Actions |  |
| N/ | N/A |

| Description of Packaging |  |
| --- | --- |
|  | 保丽龙： 16KG 800 X 800 X 20MM 2PCS面底 |
|  | 保丽龙 16kg，780 X 612 X 20 MM , 4PCS 保丽龙 12kg，620 X 120 X 40 MM , 2PCS |
|  | 保丽龙 12kg，450 X 250 X 155 MM , 1PCS |
|  | 保护板 800 X 800 X 5 MM 1PCS 夹板 100 X 100 X 5 MM 4PCS 打胶打钉固定在整板四角 |
|  | 硬纸护边 630 X 50 X 50 X 5 MM 4PCS |

| Drop Test Sequence |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drop No. (1st seq.) | Drop Height (inch) |  |  |  | Test Specimen |  | Drop No. (2nd seq.) | Drop Height (inch) |  |  |  | Test Specimen |
|  | <32kg (70lbs) |  | 32-70kg (70-150lbs) |  | Standard, Flat, Elongated |  |  | <32kg (70lbs) |  | 32-70kg (70-150lbs) |  | Standard, Flat, Elongated |
| 1 | 18 |  | 12 |  | Edge 3-4 |  | 10 | 18 |  | 12 |  | Edge 3-4 |
| 2 | 18 |  | 12 |  | Edge 3-6 |  | 11 | 18 |  | 12 |  | Edge 3-6 |
| 3 | 18 |  | 12 |  | Edge 4-6 |  | 12 | 18 |  | 12 |  | Edge 1-5 |
| 4 | 18 |  | 12 |  | Corner 3-4-6 |  | 13 | 18 |  | 12 |  | Corner 3-4-6 |
| 5 | 18 |  | 12 |  | Corner 2-3-5 |  | 14 | 18 |  | 12 |  | Corner 1-2-6 |
| 6 | 18 |  | 12 |  | Edge 2-3 |  | 15 | 18 |  | 12 |  | Corner 1-4-5 |
| 7 | 18 |  | 12 |  | Edge 1-2 |  | 16 | 36 |  | 24 |  | Most critical or damage-prone flat orientation |
| 8 | 36 |  | 24 |  | Face 3 |  | 17 | 18 |  | 12 |  | Standard: Face 3 on hazard Flat or Elongated: Face 2 on hazard |
| 9 | 18 |  | 12 |  | Face 3 |  |  |  |  |  |  |  |
| Additional drop/impact tests |  |  |  |  |  |  |  |  |  |  |  |  |
| Flat |  |  |  |  |  |  | Elongated |  |  |  |  |  |
| Rotation Edge Drop |  |  |  |  |  |  | Rotation Edge Drop |  |  |  |  |  |
| Height (in.) |  | Specific edge |  |  |  |  | Height (in.) |  | Specific edge |  |  |  |
| Support height: 4 in Drop height: 8 in |  | 1. One of the longest face 3 edges |  |  |  |  | Support height: 4 in Drop height: 8 in |  | 1. One of the longest face 3 edges |  |  |  |
|  |  | 2. Next longest edge radiating 90° from the edge just tested |  |  |  |  |  |  | 2. Next longest edge radiating 90° from the edge just tested |  |  |  |
|  |  | 3. The opposite edge tested in Sequence 2 |  |  |  |  |  |  | 3. The opposite edge tested in Sequence 2 |  |  |  |
| Full Rotational Flat Drop |  |  |  |  |  |  | Full Rotational Flat Drop |  |  |  |  |  |
| Rest on a rigid surface |  |  |  | Impact face |  |  | Rest on a rigid surface |  |  |  | Impact face |  |
| Smallest face |  |  |  | Face 3 |  |  | Smallest face |  |  |  | Face 3 |  |
| Nest largest face |  |  |  | Face 3 |  |  | Nest largest face |  |  |  | Face 3 |  |
| Concentrated Impact |  |  |  |  |  |  | Bridge Impact |  |  |  |  |  |
| Drop height (in) |  |  |  | 16 in |  |  | Drop height (in) |  |  |  | 16 in |  |