:Emmett Swivel

Counterstool

| P1 |  |  | CHAIR SEAT |  | 1PC |  |
| --- | --- | --- | --- | --- | --- | --- |
| P2 |  |  | SWIVEL |  | 1PC |  |
| P3 |  |  | CROSSING SEAT |  | 1PC |  |
| P4 |  |  | LEGS |  | 4PCS |  |
| P5 |  |  | FOOTREST |  | 4PCS |  |
| H1 | BOLTS DIA:5/16"x2-3/8" |  |  |  | 8PCS |  |
| H2 | LOCK WASHER |  |  |  | 8PCS |  |
| H3 | FLAT WASHER |  |  |  | 8PCS |  |
| H4 | BOLTS DIA:1/4"x9/16" |  |  |  | 4PCS |  |
| H5 | BOLTS DIA:1/4"x1" |  |  |  | 4PCS |  |
| H6 | FLAT WASHER |  |  |  | 8PCS |  |
| H7 | SCREW |  |  |  | 8PCS |  |
| H8 | ALLEN WRENCH 1/4" |  |  |  | 1PC |  |
| H9 | SCREWDRIVER |  |  |  | Not Provided |  |

Step 1. Assemble the chair on a flat, soft surface such as a carpet or cloth prevent scratching the finish. Insert legs(P4) to the crossing seat frame(P3), lock the legs to the seat frame using allen wrench (H8)to loosely tighten bolts (H1),lock washers(H2),flat washers(H3) as illustration below.

Step 2. Insert footrest(P5) to the legs(P4),lock the footrest to the legs with screws(H7) and tighten all screws securely with screw driver(H9) as illustration below.

Step 3. Turn the whole leg frame, upright on the surface tighten all bolts securely with allen wrench.

Step 4. Attach the swivel(P2) to the leg frame with bolts(H4) and flat washes(H6), tighten all bolts securely with allen wrench(H8) .

Step 5. Attach the swivel(P2) to chair seat(P1) with bolts(H5),flat washes(H6) and tighten all bolts securely with allen wrench(H8) .

Step 6. Turn chair upright on a level surface. Adjust levelers (as drawing) as necessary.